# Process Details and Feasibility — Updated

Revised October 14, 2026 — reconciled against the current codebase.

## Summary of Changes

- Control Plan (all 4 sheets) is live, not “on hold.” It runs today as part of the standard generation pipeline whenever the Control Plan output is requested.

- PFD/Control Plan CAD geometry: cadquery-ocp (OpenCascade) is the primary engine, tried first and used in production. FreeCAD is a fallback only, and it does not run at all on the Linux production server.

- Vision AI model corrected: gemma4:31b-cloud (via Ollama) or gemini-2.0-flash (via Gemini), not Qwen-VL — and it's a last-resort tier (Tier 3), not the primary extraction method.

- Checking Aids and PDI Report are not implemented anywhere in the codebase today — recommend changing their status from “YES” to “Planned” until built.

- Welding Check Sheet confirmed real, with an added note that it currently uses one shared global workbook rather than being scoped per user/part.

- Fixture PM Master Plan description expanded to reflect its actual day-wise (1–31) scheduling behavior.

Rows highlighted in the “Change Notes” column below were updated from the prior version; blank Change Notes rows are unchanged and were verified as still accurate.

| Document | Dynamic | Required Documents | Our Process | Accuracy | Technical Challenges | Change Notes (vs. prior version) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. BOM |
| BOM | YES | Master Data Sheet Product 2D PDF drawings | Data Extraction: pandas parses the source feasibility Excel/CSV to extract part numbers, names, quantities, thicknesses, grades, and manufacturing processes. Isometric View Location — 3-tier cascade (updated): Tier 1 — Deterministic: searches the drawing PDF (PyMuPDF) for a “REFERENCE ISOMETRIC VIEW” caption; if found, crops geometrically with no AI call at all. Tier 2 — Computer Vision: if no caption, OpenCV contour detection scans the right portion of the page for the largest non-border shape. Tier 3 — Vision AI (last resort only): draws a lettered/numbered grid over the page and asks the active vision model to return the grid cell bounding the isometric view. Image Optimization & Injection: OpenCV thresholding, contour detection and dilation surgically crop the part and remove surrounding dimension lines/text, then openpyxl writes the metadata and embeds the cropped image into the standardized BOM template. | Data – 100% Images – pending broader multi-part testing | NO | Vision model name corrected — was listed as Qwen-VL; the deployed model is gemma4:31b-cloud (Ollama) or gemini-2.0-flash, switchable via AI_PROVIDER. Added Tier 1 (text search) and Tier 2 (OpenCV) steps — these run ​before​ AI and resolve most drawings without any AI call; the original description jumped straight to the AI step. |
| 2. PFD |
| 1 – Sub Assy Index | YES | Master Data Sheet Product STP files | Resolves assembly dependencies and sub-assembly/phantom-part relationships from the feasibility data into an operation → parts routing matrix. CAD geometry extraction — now two-tier (updated): Primary: cadquery-ocp (OpenCascade/XCAF) reads the STEP assembly tree directly in-process — no external binary required, and this is what actually runs on the Linux production server. Fallback (Windows dev only): only if the OCP path fails, a FreeCAD background CLI subprocess is used instead. FreeCAD is not available on the Linux production deployment, so this fallback is inactive in production. Rendering: each extracted stage is re-imported with CadQuery, exported to SVG with hidden-line removal, rasterized via PyMuPDF, and cleaned with OpenCV before injection into the template. | Data – 100% Images – 100% | Some assemblies that previously relied on the FreeCAD fallback in Windows testing may need to succeed via OCP alone in the Linux production environment — worth spot-checking on parts known to have triggered the fallback. | Major update: original description only covered the FreeCAD CLI path. The actual primary mechanism today is cadquery-ocp (tried first); FreeCAD is a fallback that doesn't run in production at all. |
| 2 – Sub Assy | YES | Knowledgebase (Operation description, Evaluation and analysis methods) Master Data Sheet | Sheet 2 (Sub-Assembly Routing): maps operations to their corresponding fixture/part requirements, dynamically populates process steps, and uses keyword-based heuristics to classify processes (Operation, Movement, Inspection, Storage) to inject green Unicode flow symbols (●, ►, ■, ▼) in the target columns. | Data – 100% (for 1 part) Note: accuracy may vary part to part — still needs broader testing | NO | — |
| 3 – Q Sheetmetal Parts | YES | Knowledgebase (Operation description, Evaluation and analysis methods) Master Data Sheet | Sheet 3 (Sheet Metal Parts): segregates fabrication parts from the BOM based on commodity remarks, categorizes them by manufacturing process (e.g. laser cutting, bending, deburring, tapping), and dynamically maps the filtered parts list to the matching operation rows in the sheet metal flow. | Data – 100% (for 1 part) Note: accuracy may vary part to part — still needs broader testing | NO | — |
| 4 – T-BOP and Hardware | YES | Knowledgebase (Operation description, Evaluation and analysis methods) Master Data Sheet | Sheet 4 (BOP & Hardware): filters bought-out parts, fasteners and hardware items, formats them as aggregated lists (Part Number – Quantity), and injects them alongside the initial assembly operation rows with corresponding process symbols. | Data – 100% (for 1 part) Note: accuracy may vary part to part — still needs broader testing | NO | — |
| 3. PFMEA |
| 1. Assembly PFMEA | YES | Knowledgebase (Operation description/process function, Potential Failure Mode, Potential Effect(s) of Failure, Potential Cause(s) of Failure, Current Controls Prevention, Current Controls Detection) Master Data Sheet Generated PFD | Operation & Function Mapping: extracts assembly operations (e.g. OP-10A, OP-20) and process functions from the feasibility matrix, mapping them against a structured knowledge-base dictionary containing matching failure modes, effects, severities and controls. Layout Structuring: dynamically inserts green-colored operation headers and structures data rows, automatically merging duplicate cell ranges (Operation, Function, Failure Mode, Effects) to eliminate redundant text blocks. RPN Calculations: generates formula-based Risk Priority Number cells (=E*H*K per row) and appends a standardized abbreviation footer aligned precisely under the final operation. | Data – 100% (for 1 part) Note: accuracy may vary part to part — still needs broader testing | Severity, Occurrence and Detection ranking/scoring is still manual — it needs product domain experience and on-premises evaluation, which the tool does not automate. | — |
| 2. Sheetmetal Part PFMEA | YES | Knowledgebase (Operation description/process function, Potential Failure Mode, Potential Effect(s) of Failure, Potential Cause(s) of Failure, Current Controls Prevention, Current Controls Detection) Master Data Sheet Generated PFD | Fabrication Failure Dictionary: cross-references component data with the sheet-metal failure-mode dictionary to populate failure modes specific to sheet metal fabrication (laser cutting, bending, deburring, drilling). Text Wrapping & Height Tuning: dynamically calculates cell wrapping via a text-length estimator to set custom row heights, keeping multi-line failure/cause/control entries fully visible. Process Flow Sequence: generates sequential step blocks (OP-10, OP-20, etc.) with automatic cell merging for shared process descriptions, followed by standard quality guidelines and signature footers. | Data – 100% (for 1 part) Note: accuracy may vary part to part — still needs broader testing | Severity, Occurrence and Detection ranking/scoring is still manual — it needs product domain experience and on-premises evaluation, which the tool does not automate. | — |
| 3. BOP Part PFMEA | YES | Knowledgebase (Operation description/process function, Potential Failure Mode, Potential Effect(s) of Failure, Potential Cause(s) of Failure, Current Controls Prevention, Current Controls Detection) Master Data Sheet Generated PFD | Bought-Out Parts Analysis: isolates standard bought-out parts and hardware using the BOP failure-mode dictionary to capture risk profiles such as transport damage or material mix-ups. Template Cloning: replicates formatting and style details (fonts, borders, fills, columns) from the original template via helper styling functions, for structural consistency. Standard Control Integration: appends standard prevention and detection controls with severity/occurrence ratings, concluding with an abbreviation key block snapped to the last active row. | Data – 100% (for 1 part) Note: accuracy may vary part to part — still needs broader testing | Severity, Occurrence and Detection ranking/scoring is still manual — it needs product domain experience and on-premises evaluation, which the tool does not automate. | — |
| 4. Control Plan |
| 1. Sub Assy Index | YES | Master Data Sheet Product STP files Generated PFD | Reuses the same CAD image set generated for PFD Sheet 1 (cadquery-ocp primary / FreeCAD fallback, see Section 2) for template injection. | Data – 100% Images – 100% | NO | Status corrected: was marked “YES (ON HOLD)” — this sheet is now live in the generation pipeline. |
| 2. Assy Sub Assy | YES | Master Data Sheet Generated PFD Knowledgebase (Product, Process, Special Characteristics, Spec/Tolerance, Evaluation/Measurement, Size, Freq, Control Method, Responsibility, Reaction Plan) | Feasibility File: defines the high-level operation mapping — identifies the main assembly groupings (OP-10A, OP-30, etc.) that structure the control plan. Template File: provides the base visual layout — header rows, logos, and the “Note:” footer, preserving the original document's exact structure. Generated PFD File: the primary source of truth for assembly sequence — supplies the OP-label, parts/sub-assemblies list for the green header blocks, and the Operation No., Description and Machine/Device/Jig/Tools for each step. Knowledge-base Dictionary: maps assembly operation descriptions to standardized control-plan parameters — Product, Process, Special Characteristic Class, Evaluation/Measurement Technique, Size, Freq, Control Method, Responsibility, and Reaction Plan. | Data – 100% | NO | Status corrected: was marked “YES (ON HOLD)” — this sheet is now live and generated as part of every requested Control Plan run. |
| 3. Q Sheetmetal Parts | YES | Master Data Sheet Generated PFD Knowledgebase (Product, Process, Special Characteristics, Spec/Tolerance, Evaluation/Measurement, Size, Freq, Control Method, Responsibility, Reaction Plan) | Feasibility File: extracts the foundational sheet metal parts — Part No., Description, Thickness/Size, Material Grade, and the Operation No. that dictates the “This Part assembles in OP-X” block. Template File: provides the base visual layout — header rows, logos, and the “Note:” footer, preserving the original document's exact structure. Generated PFD File: looks up the specific operation sequence for each part — Operation No., Description, and Machine/Device/Jig/Tools, filtered by part number. Knowledge-base Dictionary: maps operation descriptions from the PFD to standardized control-plan parameters, and dynamically tracks “Storage” operations to apply “Storage 2” parameters on the final step. Images Folder: fetches the {Part_No}_clean.png generated earlier in the BOM step and embeds it centered in the green header block for that part. | Data – 100% Images – 100% | NO | Status corrected: was marked “YES (ON HOLD)” — this sheet is now live. |
| 4. T-BOP and Hardware | YES | Master Data Sheet Generated PFD Knowledgebase (Product, Process, Special Characteristics, Spec/Tolerance, Evaluation/Measurement, Size, Freq, Control Method, Responsibility, Reaction Plan) | Deterministic data-injection and layout-preservation engine (openpyxl) that synthesizes the BOP & Hardware control plan sheet from structured knowledge-base metadata, with a memory-buffered state-tracking system that re-draws the legacy footer (cell styles, row heights, multi-column merge ranges) on every rewrite, plus adaptive multi-row merging and content-aware row-height logic so multi-line technical descriptions don't clip. | Data – 100% | NO | Status corrected: was marked “YES (ON HOLD)” — this sheet is now live. |
| 5. List of Tooling |
| Fixture Master List | YES | Master Data Sheet | Embedded Image Extraction: scans the feasibility workbook with openpyxl for embedded floating images, using a fuzzy row-offset search to match each image to its nearest row's “Fixture No.” Fixture Details Parsing: extracts fixture metadata — operation numbers, fixture names/numbers, part names, welding source types, and manufacturer (“make”). Excel Injection: copies the master Tooling List template to preserve formatting, injects header details, writes the tabular fixture records, and inserts the extracted fixture photos at original scale, adjusting row heights/column widths dynamically. | Data – 100% | NO | — |
| Fixture PM Master Plan | YES | Master Data Sheet | Builds a monthly fixture preventive-maintenance schedule from the master data sheet, with day-wise tracking (1–31) injected into a fresh copy of the PM plan template. | Data – 100% | NO | Expanded description — original entry (“All the details was extracted from master data”) didn't capture the day-wise (1–31) PM scheduling behavior. |
| 6. Part Fitment Check Sheet |
| Part Fitment Check Sheet | YES | Master Data Sheet | BOM Data Extraction: reads the feasibility matrix to compile a structured BOM of all child component part numbers, descriptions and assembly quantities. Dynamic Row Replication: replicates template rows for each part, copying border styles, cell fonts, and the template's merged-cell range patterns. Component Image Inlay: maps each part number to its corresponding cleaned CAD drawing image (reused from the BOM step), inserts and centers it in column D, adjusts row heights, and pulls the “REMARK” footer upward to snap under the last data row. | Data – 100% Images – 100% | NO | — |
| 7. Checking Aids |
| Checking Aids | — | Master Data Sheet (gauges & instruments data, if provided) | Not yet implemented in the current pipeline. If gauge/instrument data is supplied in the master data sheet, this is feasible to automate — remains a planning item, not a shipped feature. | N/A | NO | Status corrected: was marked “YES.” No Checking Aids generator exists anywhere in the current codebase — confirmed by direct search. Recommend marking this “Planned,” not “YES,” until it's built. |
| 8. PDI Report |
| PDI Report | — | Master Data Sheet | Not yet implemented in the current pipeline. Open question carried over from the original doc: whether critical parts are identified automatically or manually — still unresolved, and no PDI Report generator exists in the codebase today. | N/A | NO | Status corrected: was marked “YES.” No PDI Report generator exists anywhere in the current codebase — confirmed by direct search. Recommend marking this “Planned,” not “YES,” until it's built. |
| 9. Welding Check Sheet |
| Welding Check Sheet | YES | CAD – Ballooned File PDF | A standalone cropper tool (separate from the main generation pipeline): the operator drag-selects a crop region from the ballooned drawing PDF in-browser, and it's injected directly into the welding checksheet workbook — no manual copy-paste. A “set reference image” endpoint runs the same isometric-extraction pipeline used by BOM generation to auto-populate a reference view, independent of any BOM job. | — | Extracting weld symbols directly from the CAD file is not accurate or consistent (original note — still applies). Architectural note: this tool writes into one single shared workbook used by every user, not a per-part or per-session file — worth scoping per-user/per-part before multiple people use it concurrently. | Added architectural note on the shared single-workbook design — relevant if this tool will see concurrent multi-user use. |